User manual

Settler Unitouch

Hand terminal

Software version 4.17

Congratulations with your new acquisition, the Settler® Unitouch hand terminal!

In front of you, you will see the user manual for the Settler® Unitouch Hand terminal. To use the Settler® Unitouch hand terminal to the fullest, you will find a comprehensive description of the functions of this unique piece of equipment.

The Settler® Unitouch hand terminal is:

- Fast

- Reliable

- Easy to use

- Obvious

In between of the text you will find some screenshots, which will help you to get a better understanding about the describes menu functions.

You can quickly search the manual with help of the contents on the next page. If you have any questions left about the Settler® Unitouch Hand terminal. You can always contact your Settler® Unitouch dealer.

Note:

It is possible that some of features are available on your Settler® Unitouch Hand terminal. There are several versions of the hand terminal available and it is also related to the software license which you are using.

# Contents

Contents	3

1. Introduction	4

1.1 Front of the hand terminal	4

1.2 Back of the hand terminal	6

1.3 Top and bottom of the handheld	7

2. The battery	7

2.1 Install the battery	7

2.2 Remove the battery	7

2.3 Charge the battery in the hand terminal	8

2.4 Charge the battery in the battery compartment	8

3. Writing	9

4. Stand-by	10

5. Setting the language	10

6. Logging in as a waiter	11

7. Logging out as a waiter	12

8. Opening a table	13

9. Opening a customer	14

10. Opening a room	15

11. Opening a ‘tag’	16

12. Entering orders on a seat level	17

13. Enter a new order	18

14. Free text input	20

15. Settling a table	21

15.1 tBill and a payment afterwards.	21

15.2 Pay the bill immediately	21

15.3 Book the bill on a customer level	22

15.4 Split payment of the bill	22

Split a table	22

Split a table and settle it immediately	22

Split to a customer or room	22

Move a complete table	22

Move to a customer or room	22

16. Overviews	23

17. Settings	24

18. Troubleshooting	26

Notes:	27

# 1. Introduction

You can control the hand terminal by use of a stylus which you can find a the right backside of the hand terminal. Use this to touch the screen, we advise to only use this stylus, other pens can damage the screen.

At the bottom of the touch screen you will see a dark bar, which is used to enter letters, numbers and special characters.

## 1.1 Front of the hand terminal

1	Power switch

2	Led indicator

3	LCD (touch screen)

4	Connector

5	Scan trigger

6	Right trigger

7	Left trigger

8	Navigation

9	Home

10	Enter

11	Brightness/contrast

12	Function keys

Power switch	This button will switch the hand terminal on or off. A short push is enough to do this. When you hold this button for a longer time, you can turn the backlight on or off.

LED indicator	During the charging of the unit, this LED will light up, when this LED is green the hand terminal is fully charged, when this LED is orange the hand terminal is charging. When the LED is red, an error has occurred.

LCD	The LCD touch screen is used for entering the orders.

Connector	The connector connects to the charging connector of you cradle.

Scanner		Key that is used for scanning of the products.

Left trigger		Not used.

Right trigger 	Not used.

Navigation key	The navigation key can be used to scroll between the available product pages and the products.

Left & right is used for scrolling between the pages

Up & down is used for scrolling between the products

Home	To end an order.

Select/enter		Not used.

Brightness	This button is used to modify the brightness of the LCD screen.

F1	Use this key to set the hand terminal into stand by. The backlight will be turned off and the screen is blocked. To reactivate the screen you can press the F1 key again or draw a line from the bottom left of the screen to the top right.

F2 F3 F4	Used for closing an order.

The dark part at the bottom of the screen is the entry pad. Here you can write letters, numbers and special characters. You can find more information about writing in chapter 3 of this manual.

## 1.2 Back of the hand terminal

- Battery door

- Battery screw

- Reset button

- Buzzer

- IrDA window

- Stylus

- Stylus slot

- Handstrap connector

## 1.3 Top and bottom of the handheld

- Barcode scanner

- Power switch

- Syncing / Charging connector

# 2. The battery

## 2.1 Install the battery

- To access the battery compartment, turn the locking-screw counterclockwise to open the latch and

- remove battery cover.

- Position the battery lift strap against the back of battery well, ensuring tab extension is on left side.

- Place the rechargeable battery in and snap into place.

- Replace the battery cover by positioning the top prongs first and sliding into place. Tighten locking screw.

- by turning clockwise.

## 2.2 Remove the battery

- To access the battery compartment, turn the locking-screw counterclockwise to open the latch and

- remove battery cover.

- Pull the lift strap and the battery will pop out.

## 2.3 Charge battery in the hand terminal

- Connect the power supply to the cradle.

- Connect the line cord to the appropriate power supply.

- Plug in the power supply.

- Insert the hand terminal into the cradle.

Important: Proper insertion is required to ensure connection. Be sure to insert the device as follows:

- Place the hand terminal in cradle well (seating the device without trying to join connectors).

- Tilt-rotate the hand terminal forward (bringing the top end of the unit forward) as you apply downward pressure to fully insert in cradle well, joining connectors.

- The LED will illuminate to confirm connection (orange indicates charging, green indicates fully charged and red indicates fault).

- The battery is fully charged in approximately two hours.

## 2.4 Charge battery in the battery compartment

- At the backside of the cradle is a compartment where you can charge a spare battery.

- Place the battery in the compartment with the Janam logo to the right.

- Push down the battery

- The LED will illuminate to confirm connection (orange indicates charging, green indicates fully charged and red indicates fault).

- The battery is fully charged in approximately two hours.

- To remove the battery, place your index finger in the gap between the battery and the cradle (at the side of the LED) and gently pull the battery upwards. The battery will pop out of the compartment.

# 3. Writing

Note

Only use the stylus which is delivered together with the hand terminal, use of other styluses or pens can severely damage the screen. Extra styluses can be ordered at your Settler® Unitouch dealer.

Use the gestures shown below to write the letters or numbers on the hand terminal.

4. Stand-by

The hand terminal will automatically go to stand-by when it is not used for a while. This is done to extend the battery life, so that you can use the hand terminal for a longer time without recharging it. To switch the hand terminal back on, press the power switch for 1 second.

# 5. Setting the language

Before you can use the language it is helpful if the language is set right. The default language is Dutch.

The following languages are available

- Dutch 	Nederlands

- English	English

- German	Deutsch

- Swedish	Svenska

- Spanish	Español

When you have selected the language, you can use the Settler® Unitouch hand terminal in your own language.

# 6. Logging in as a waiter

Every staff member who has a key for the Settler® Unitouch POS, can log in to the hand terminal. To make sure that a staff member can log in to the hand terminal, he will need a login code. The default login code is ‘1’. You can change the login codes in the BackOffice program or you can contact your Settler® Unitouch dealer if you need help with this.

To log in you will need to take the following steps:

- Turn on the hand terminal by pressing the power switch at the top.

- The login screen will be shown.

- On the dotted line below access code will need to fill in your personal entry code.

- Do this by writing the numbers in the numeric part of the input field or you can use the numeric keyboard (see chapter 3)

- When you have written a wrong number, you can erase it by drawing horizontal line from right to left in the input field.

- Press the login button to log in to the hand terminal.

When you have entered the right entry code, you are now logged into the hand terminal. You are also logged into the Settler® Unitouch cash register, now you can fast and wireless:

- Enter all your orders

- Create customer bills

- Check the unsettled items

- Split and move

# 7. Logging out as a waiter

When you are temporary are not entering orders, you can easily switch off the program by pressing on the power switch of the hand terminal.

When you press the power switch again you will see, that you are still logged into the system.

If you want to log out of the system, press the Logout button. You are now logged out of the system.

# 8. Opening a table

On the hand terminal, you can just like on the cash register, register orders on a table level. You will need to enter a table number before you can enter an order.

When you know the number of the table, you can enter this number and press on Open.

To open a table follow the following steps:

When the table number is known:

- When you know the table number, just enter this number with the numeric keys, the entered numbers will appear on the dotted line behind ‘Table’.

- Press the Open button to open the table.

When the table number is not known:

- Press the Open button without filling in a table number.

- A table screen will pop-up, which is similar as on the cash register. 
An example is shown to the right.

- When there are multiple table screens, you can switch between these by pressing the arrow keys in the bottom right corner.

- Simply select the table you wish to open and the table will be opened.

- When you don’t want to open a table, press the Back button.

# 9. Opening a customer

It is possible to open a customer instead of opening a table, entering orders on a customer level is also possible on the hand terminal.

When you press the arrow in front of table a menu will drop down where you can choose customer.

Select customer and press on open. An alphabetical list will be displayed with all the available customers.

When you enter a letter or a name in the input field, the hand terminal will jump to this letter or name.

If you found the customer in the list, select the customer and click the select button.

Note:

To use this option, a customer database must be available in the cash register. In the cash register must be set up, that all the customers must be send to the hand terminal.

# 10. Opening a room

Besides opening a table or customer, there’s also the option to enter orders on a room level.

When you press the arrow in front of table, you can choose for room. If the room number is known, you can fill it in with the numeric buttons and press open.

If you own a hotel and you are using the reservation system of Hotelconcepts, then there is an option to connect the two systems together. All the orders that are entered in the cash registers are added to the account of the room.

When a room is opened, there is also a name check, to make sure no errors are made.

Fill in a room number and the system will ask, guest Mr. Brown?

Accept to book the order on this guest or cancel.

# 11. Opening a ‘tag’

When you are using tags, wristbands or member cards; there is an option to scan these directly with the hand terminal. When you press the scan tag button, a time bar will be shown in which you will need to scan the tag.

When this is done, al the orders will be registered to this personal tag.

Depending of the settings, there is also an option to enter a table first and scan the tag afterwards.

Important

This is NOT a standard feature! To make use this feature your hand terminal must be equipped with a special RFID reader. Contact your Settler® Unitouch dealer for more information.

# 12. Entering orders on a seat level

There is also the options to enter new orders on a seat level. With this a group of 4 persons can sit at 1 table, but they can use separate accounts.

You will also receive an order ticket with a note for who is what consumption.

When you want to split a payment, you press the Move/Split button and select the table. The screen shown below will appear.

In the left column you see the different ranks.

Behind each consumption is shown on which chair this consumption is registered.

By using the arrow keys, you can select which consumptions you want to pay.

Select a line and press the right pointing arrow. The 0 in the second column will change into a 1. This way you can see which products are selected. To undo this selection, press the left pointing arrow

When you press the split/bill button, the selected products can be paid. The other products are still registered to the table.

# 13. Enter a new order

After the selection of a table, customer or a room. You can enter the wanted order.

The following screen is shown:

As you can see, the screen consist of two parts. At the left side you see the available product pages and the right you see the products that belong to these pages.

To quickly navigate trough these screens you can use the navigation key of the hand terminal.

The left and right arrow is used to browse the product pages. The up and down arrow is used to browse the products at the right part of the screen.

You can easily choose the products by tapping on them with the stylus.

When your order is complete, you can leave this screen by tapping the exit symbol at the bottom right.

The below screen will be displayed. When this order is ok, you can accept this order by pressing the End button. This order will be booked on the selected table, room or customer. At the same time the bar or kitchen tickets will be printed.

Short description about the available keys:

PAY	Used when the customer wants to pay this 	order immediately.

PLU	Used to add product to this order by entering 	a PLU number.

END	Used to end the order and send it to the bar 	or kitchen.

DEL	Select a product from the list and press this 	button to delete it.

M+	To zoom into a choice menu

M- 	To zoom out of a choice menu

Behind the ordered drinks and dishes, you also see two columns of numbers; the first numbers shows the rank of this product, the second number the seat number.

At the lower part of the screen there are some more function keys.

Qty, Rank and Seat use plus and minus keys to the right of them.

Qty. +	To add up a earlier entered consumption

Qty. -		To deduct an earlier entered consumption

Rank +	To add up the rank

Rank -	To lower the rank

Seat +	To add up the seat number

Seat -		To lower the seat number

When you forgot to enter a product, you can return to the product selection screen by tapping the Multi button.

When this order is entered, the color of the table will change to red on the hand terminal as well as on the cash register.

# 14. Free text input

Most systems are designed in such a way, that there are multiple pages with kitchen messages, bar messages or any other notes. When you want to use a free text input, a message that is not default on your system, you can use the following methods.

- Open the desired table and select the product page with the free text product.

- Select the free text product, a screen will pop up.

- Enter the message by using the input field or the keyboard the message.

- Select on which printer this message should be printed, the kitchen printer or the bar printer. Or if this message should follow the product that has been entered before this message.

When there is no free text PLU available you can also use the following method.

- When all the product are entered in the product selection screen, tap the exit button.

- The order overview screen is displayed.

- Select the product at which you want to attach the message.

- Tap on ‘order’, at the top left of the screen and select remark in the menu.

You can use the ‘Nr. Persons’ function in this menu to set the number of persons afterwards.

When de options function is selected, the screen shown to the right is displayed.

In this screen you can add a free text to this product or set this product to be free of charge.

The number of ordered items of this product, the rank and price is also displayed

# 15. Settling a table

There are several ways to settle a table:

## 15.1 tBill and a payment afterwards.

- Enter the table number.

- Tap the pay button, the Pay screen will be opened.

- When the table number is not known

- Tap the pay button.

- The table screen is shown, where you can choose the desired table.

- Choose a tBill by tapping the tBill button.

## 15.2 Pay the bill immediately

To settle the table immediately:

- Enter the table number.

- Tap the pay button, the Pay screen will be opened.

- When the table number is not known.

- Tap the pay button.

- The table screen is shown, where you can choose the desired table.

- Select the desired payment method by pressing the arrow key underneath ‘payment’.

- Fill in the received amount of cash (use the comma or point), and tap the Enter button.

- The return amount of cash is displayed.

If you want an overview of the bill, you can press the view button. You can close this view by tapping the cross at the bottom right.

- Select the receipt box when you want a bill to be printed.

- Tap the paid button to settle this bill or back to cancel.

## 15.3 Book the bill on a customer level

When a customer desires to pay this bill later, there is an option the write this unsettled table to a customer account. This customer must be already be available in the system. When not, this customer must be created first in the BackOffice or in the cash register.

## 15.4 Split payment of the bill

### Split a table

- Fill in the table number at the table selection screen and press the move/split button.

- Select the product that needs to be split and press the right pointing arrow.

- In front of this product the 0 changes into a 1. When you want this product multiple times tap this product again, the 1 changes into a 2 (two times selected), every tap this number is add up by one.

- You can tap the left pointing arrow to undo a selection.

- When al the desired products are selected, press the ‘move’ button.

- Enter the desired table number and tap on Ok. Tap on ‘back’ to return to the table selection screen.

- The selected products are now moved to the selected table.

### Split a table and settle it immediately

- Follow the procedure above, but instead of tapping the Move button, tap the Split/Bill button.

- Follow the payment procedure.

### Split to a customer or room

- Use the same procedure as splitting a table, but instead of choosing a table use the arrow key next to table to select customer or room.

### Move a complete table

- Do not select a product

- Press the move button

- Enter  the new table number and tap Ok

- The complete order is moved to this table.

### Move to a customer or room

- Use the same procedure as moving a table, but instead of choosing a table use the arrow key next to table to select customer or room.

# 16. Overviews

Just like on the cash register, it is possible to get several overviews on the hand terminal. These are the unsettled items separated into unsettled tables, unsettled rooms and unsettled customers.

To receive an overview of the unsettled items, press the browse button in the table selection screen.

Select table to get an overview of the unsettled tables, room for an overview of the unsettled rooms or cust. to get an overview of the unsettled customers.

In the above example, a overview of the unsettled tables is shown. When a table  is selected, you can open this table or settle this table by pressing the pay button.

To return to the table selection screen, tap the Back button.

The ranks that have been sent to the cash register can also be viewed. At the right side the number 99/06 is shown. From this you can see that rank 6 is the highest rank that has been ordered at this time. The number 99 shows till what rank the order has been sent to the kitchen. When this number is 05, you can press the rank button the send the next rank to the kitchen.

In this case the waiter decides when the rank (dish) the kitchen staff should made for the selected table.

# 17. Settings

On the hand terminal there is an option to change some settings yourself.

IMPORTANT

It is NOT advised to make changes without contacting the service department of your Settler® Unitouch dealer. When some settings are changed, it is possible that the hand terminal is not works like it did before.

Data Entry:	The type of data entry for the products. You can choose from Look up level or PLU level. Default this is set to Look up level.  The selected method is shown in blue.

Multi:	When this box is enabled, multiple choices can be made at the product selection screens. When this is disabled only one selection at a product page can be made. Default this box is enabled

Cash Reg:	This is the node number of the Settler® Unitouch cash register. When an order is entered this order will be printed thru one of the cash registers. Default this is Cash reg 1. (node 1)

Bon:	When you use multiple Bill lay-outs, you can select the type of bill in this drop down menu. If you have a slip bill printer, you can select a slip bill here.

TxtPlu:	The PLU number of the free text input, this is used to send free text messages. Default this is set  on 1999 or 2003.

K:			The number of the kitchen printer.

B:			The number of the bar printer.

Split AccNr:	The table number that is used temporary when splitting or moving a account. Use a table number that is not available on your table screen. For example table number 900.

Auto Off:	The time in seconds after which the hand terminal will automatically be turned off. This can be set between 10 seconds and 600 seconds.

Nr.Persons:	Select this box if you want to enter the number of persons for each table. This can be used as an extra check, when there are 4 persons at a table, usually there are 4 drinks to be ordered.

Mandatory:	When this box is selected, entering the number of persons at a table is mandatory. This way the waiters will know for how many persons the table should be set.

Large Font:	Default a small font is used by the hand terminal, this way more text can fit on the screen.

When this font is too small, you can use a larger font by selecting this box. Keep in mind that not all the text can fit in the screen if this box is selected.

Start Sync:	When there have been changes in the programming of the Settler® Unitouch cash register and these changes are not loaded into the hand terminal. Tap this button and the hand terminal will sync with the cash register.

DL Images:	Use this button to download the backgrounds of the table screens to the handheld. Now you have the same backgrounds on your hand terminal as on your Settler® Unitouch cash register.

# 18. Troubleshooting

| Problem | Solutions |
| --- | --- |
| The screen is dark, the program can be viewed on screen. | Press the F1 key to turn on the backlight. |
|  | Press the brightness key, then press and hold the right arrow key of the navigation keys. |
|  | Press and hold the power switch for 5 seconds |
| The screen is dark, nothing to see on screen | Press the power switch |
|  | Check if the battery door is in place |
|  | Remove the battery door, use the stylus to press the reset button. |
|  | Remove the battery door and battery, place the battery again. |
|  | Check if the battery is charged, place the hand terminal in the cradle. |
| The standby time of the hand terminal is to short. | You can set the stand by time of the screen in the settings screen. Go to the settings screen, change the Auto off time to the desired time. It can be set between 10 seconds or 10 minutes (600 seconds). Default this is set on 10 minutes. |
| The data on the hand terminal is not synced with the cash register. | Go to the settings screen, tap on the Start Sync button. The hand terminal will sync with the Cash register. |
| There is no or an old background be shown at the table screen. | Go to the settings screen, tap on the DL images button, the backgrounds will be downloaded from the Settler® Unitouch cash register. |

# Notes:

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________